### Understanding the Testing Section in Lab 3

The testing process in Lab 3 involves creating testbenches for each gate you designed (NAND2, NOR2, MUX2, XOR2, AOI3) and simulating them in Cadence ADE XL. You will measure transient responses and extract timing characteristics (Rise Time, Fall Time, Propagation Delays) for each gate.

## 1. Overview of Testing for Each Gate

Each gate requires:

- A functional transient simulation to verify logic operation.

- Timing measurements:

  - Rise Time (Time for output to go from 10% to 90% of VDD)

  - Fall Time (Time for output to go from 90% to 10% of VDD)

  - Propagation Delays:

    - T_PLH (Low-to-High delay) → Time from input falling edge to output rising edge.

    - T_PHL (High-to-Low delay) → Time from input rising edge to output falling edge.

For each gate, you will set up test cases for each possible input-to-output transition, considering the truth table.

## 2. Setting Up the Simulation in ADE XL

### Step 1: Create a Testbench for Each Gate

- Open Virtuoso Library Manager.

- Create a new schematic cell view (e.g., FML_NAND2X1_TEST).

- Instantiate the gate symbol (e.g., your NAND2, NOR2, etc.).

- Add power sources (vdd! and vss!).

- Add a capacitor from analoglib, with CAP = 100 fF.

- Label input pins (A, B, or S for MUX) and output pin (Y).

### Step 2: Configure ADE XL for Transient Simulation

- Launch ADE XL (Launch → ADE XL).

- Create a new Test View and select your testbench schematic.

- Disable APS Mode (Setup → High-Performance Simulation, choose Spectre).

- Select Model Libraries (Setup → Model Libraries):

  - Use tt (typical) corner model.

### Step 3: Apply Input Stimulus

For each gate, configure input A, B, or S as either a pulse (switching input) or DC (static input).

#### Example: NAND2 Gate (A → Y ↑ Test)

- Stimuli Setup:

  - Setup → Stimuli → Inputs

  - For A (switching input):

    - Function: Pulse

    - Voltage 1: 1V

    - Voltage 2: 0V

    - Delay Time: 100 ns

    - Rise Time: 100 ps

    - Fall Time: 100 ps

  - For B (static input):

    - Function: DC

    - Voltage: 1V

- This ensures we toggle A while keeping B = 1, so we only observe the A → Y transition.

### Step 4: Running the Transient Analysis

- Set Up Transient Analysis (Analysis → Choose → tran).

  - Stop Time: 200 ns.

  - Enable it and click OK.

- Select Outputs to Plot (Outputs → To Be Plotted → Select On Schematic).

  - Select input A and output Y.

  - Press ESC to exit selection mode.

- Click the Green Start Button to run the simulation.

- View the waveform results (Plot All → Split All Strips).

### Step 5: Extract Rise Time, Fall Time, and Delays

- Open the Calculator (Browser → Results → Open Results).

- Select Output Y, Right Click → Open Calculator.

- Use:

  - riseTime() → Measure output rise time.

  - fallTime() → Measure output fall time.

  - delay() → Measure propagation delay:

    - T_PLH (Low to High delay): Falling edge of A → Rising edge of Y.

    - T_PHL (High to Low delay): Rising edge of A → Falling edge of Y.

- Save and Name Expressions (e.g., RiseTime_Y, T_PLH_Y).

- Run the simulation again and confirm values.

## 3. Required Tests for Each Gate

For each gate, perform the following tests:

### NAND2 Gate

| Test Case | Input Toggle | Static Input |
| --- | --- | --- |
| A → Y ↑ | A toggles | B = 1 |
| A → Y ↓ | A toggles | B = 1 |
| B → Y ↑ | B toggles | A = 1 |
| B → Y ↓ | B toggles | A = 1 |
| Functional Simulation | Verify logic correctness |  |

### NOR2 Gate

| Test Case | Input Toggle | Static Input |
| --- | --- | --- |
| A → Y ↑ | A toggles | B = 0 |
| A → Y ↓ | A toggles | B = 0 |
| B → Y ↑ | B toggles | A = 0 |
| B → Y ↓ | B toggles | A = 0 |
| Functional Simulation | Verify logic correctness |  |

### MUX2 Gate

| Test Case | Input Toggle | Static Inputs |
| --- | --- | --- |
| A → Y ↑ | A toggles | B = 0, S = 0 |
| A → Y ↓ | A toggles | B = 0, S = 0 |
| B → Y ↑ | B toggles | A = 0, S = 1 |
| B → Y ↓ | B toggles | A = 0, S = 1 |
| S → Y ↑ | S toggles | A = 0, B = 1 |
| S → Y ↓ | S toggles | A = 0, B = 1 |
| Functional Simulation | Verify logic correctness |  |

### XOR2 Gate

| Test Case | Input Toggle | Static Input |
| --- | --- | --- |
| A → Y ↑ | A toggles | B = 1 |
| A → Y ↓ | A toggles | B = 1 |
| B → Y ↑ | B toggles | A = 1 |
| B → Y ↓ | B toggles | A = 1 |
| Functional Simulation | Verify logic correctness |  |

### AOI3 Gate

| Test Case | Input Toggle | Static Inputs |
| --- | --- | --- |
| A → Y ↑ | A toggles | B = 1, C = 1 |
| A → Y ↓ | A toggles | B = 1, C = 1 |
| B → Y ↑ | B toggles | A = 1, C = 1 |
| B → Y ↓ | B toggles | A = 1, C = 1 |
| C → Y ↑ | C toggles | A = 1, B = 1 |
| C → Y ↓ | C toggles | A = 1, B = 1 |
| Functional Simulation | Verify logic correctness |  |

## 4. Final Steps

- Repeat steps for each gate (NAND2, NOR2, XOR2, MUX2, AOI3).

- Fill in the test results with:

  - Rise Time

  - Fall Time

  - T_PLH, T_PHL delays

- Capture screenshots of:

  - Transient simulation waveforms

  - Extracted timing measurements

- Submit your results and datasheets.

### Let me know which part is confusing, and I can clarify it further! 😊

S