| Library Name: | cgm |  |
| --- | --- | --- |
| Cell Name: | AOI3 |  |
| Function/Truth Table: Y = (B+C).A |  |  |
| A B C |  | Y |
| 0 0 0 |  | 1 |
| 0 0 1 |  | 1 |
| 0 1 0 |  | 1 |
| 0 1 1 |  | 1 |
| 1 0 0 |  | 1 |
| 1 0 1 |  | 0 |
| 1 1 0 |  | 0 |
| 1 1 1 |  | 0 |
| Propagation Delay (path based): |  |  |
| Transition |  | Delay (s) |
| A → Y ↑ (T_PLH) |  | 1.25n |
| A → Y ↓ (T_PHL) |  | 1.326n |
| B → Y ↑ (T_PLH) |  | 1.397n |
| B → Y ↓ (T_PHL) |  | 1.633n |
| C → Y ↑ (T_PLH) |  | 1.369n |
| C → Y ↓ (T_PHL) |  | 1.682n |
| Output Rise Time (path based): |  |  |
| Transition |  | Rise Time (s) |
| A → Y ↑ |  | 2.484n |
| B → Y ↑ |  | 2.679n |
| C → Y ↑ |  | 2.652n |
| Output Fall Time (path based): |  |  |
| Transition |  | Fall Time (s) |
| A → Y ↓ |  | 2.457n |
| B → Y ↓ |  | 3.104n |
| C → Y ↓ |  | 3.234n |
| Layout Height: 1.71µm |  |  |
| Layout Width: 1µm |  |  |
| Layout Area: 1.71µm2 |  |  |
| Symbol with Port Names: |  |  |
| Schematic: |  |  |
| Layout: |  |  |
| Functional Simulation Waveforms |  |  |
| Comments/Notes: The AOI3 (AND-OR-INVERT) gate was successfully tested, confirming correct functional verification. The extracted parasitics have impacted timing, with rise times increasing compared to the pre-layout results, while fall times have improved. This behavior is due to the influence of parasitic capacitances and resistances in the extracted layout, which affect the charge and discharge paths differently. Despite these variations, the gate meets expected performance requirements and remains functionally accurate. |  |  |